实验报告 Lab 4

一、按钮测试结果

1、防止亚稳态

缺少该模块可能导致亚稳态，输出不稳定：触发器的输出可能会在0和1之间振荡，或者在很长时间内保持在一个中间电平，可能导致后续逻辑电路接收到错误的值，进而导致系统行为异常或崩溃。

2、消抖模块

按钮在按下和释放时会产生机械抖动，导致多个上升沿和下降沿。若没有模块处理抖动，可能会导致输出状态在短时间内多次切换。

3、脉宽变换模块：

无模块

有模块

脉宽变换模块负责将按钮按下的脉冲控制在一个时钟周期，避免后续导入FSM时出现意外的状态转换。

如图，在b1由0转1时，next_state由1转2，如果不把开关脉冲控制在一个周期内，就会误认为按钮一直被按下，从而发生意想不到的状态转变，如图中next_state从2意外跳变到0。

二、行为级代码

代码（见附录）

仿真（见下图）

FPGA板上结果（见下图）

| 1）初始/按下RESET | 2）按下“0” |
| --- | --- |
| 3）按下“1” | 4）按下“0” |
| 5）按下“1” | 6）按下“1” |

三、状态图 状态方程 驱动方程 状态机电路

手写状态图（见下图）：

由状态图可列出如下的状态转换表：

| 现态S2S1S0 | 次态S2nextS1nextS0next |  |
| --- | --- | --- |
|  | Y=0 | Y=1 |
| (RESET) 000 | 001 | 000 |
| (0) 001 | 001 | 010 |
| (01) 010 | 011 | 000 |
| (010) 011 | 001 | 100 |
| (0101) 100 | 011 | 101 |
| (01011) 101 | 001 | 000 |

由以上状态转换表可推导出如下的状态方程和驱动方程：

S2next= S2’S1S0Y + S2S1’S0’Y

S1next= S2’S1’S0Y+ S2’S1S0’Y’ + S2S1’S0’Y’

S0next= S2’Y’+ S2’S1 (Y’ + S0’)

Unlock= S2S1’S0

状态机电路图如下：

四、电路级代码

电路级代码（见附录）

附录 代码

1 行为级代码

module lock(

input reset,

input b0,

input b1,

input clk,

output an,

output reg a,

output reg b,

output reg c,

output reg d,

output reg e,

output reg f,

output reg g,

output led

);

wire b0_out;

wire b1_out;

onoff button_0(.clk(clk),.reset1(1),.button_in(b0),.out(b0_out));

onoff button_1(.clk(clk),.reset1(1),.button_in(b1),.out(b1_out));

wire[3:0]display;

FSM fsm1(.clk(clk),.reset(reset),.b0(b0_out),.b1(b1_out),.out(led),.display(display)

);

assign an = 1;

always @(*) begin

case(display)

3'b000 : {a,b,c,d,e,f,g}=7'b1111110;

3'b001 : {a,b,c,d,e,f,g}=7'b0110000;

3'b010 : {a,b,c,d,e,f,g}=7'b1101101;

3'b011 : {a,b,c,d,e,f,g}=7'b1111001;

3'b100 : {a,b,c,d,e,f,g}=7'b0110011;

3'b101 : {a,b,c,d,e,f,g}=7'b1011011;

default : {a,b,c,d,e,f,g}=7'b1111110;

endcase

end

endmodule

module FSM(

input clk,reset,b0,b1,

output out,

output [3:0] display

);

parameter S_RESET = 0; parameter S_0 = 1;

parameter S_01 = 2; parameter S_010 = 3;

parameter S_0101 = 4; parameter S_01011 = 5;

reg [2:0] state, next_state;

always@(*) begin

if(reset==0) next_state = S_RESET;

else case (state)

S_RESET: next_state = b0 ? S_0  :b1 ? S_RESET:state;

S_0: next_state = b0 ? S_0  :b1 ? S_01:state;

S_01: next_state = b0 ? S_010  :b1 ? S_RESET:state;

S_010: next_state = b0 ? S_0  :b1 ? S_0101:state;

S_0101: next_state = b0 ? S_010  :b1 ? S_01011:state;

S_01011: next_state = b0 ? S_0  :b1 ? S_RESET:state;

default: next_state = S_RESET;

endcase

end

always@(posedge clk) state<= next_state;

assign out = (state == S_01011);

assign display = {1'b0,state};

endmodule

module display(

input [2:0]din,

output an,

output reg a,

output reg b,

output reg c,

output reg d,

output reg e,

output reg f,

output reg g

);

assign an = 1;

always @(*) begin

case(din)

3'b000 : {a,b,c,d,e,f,g}=7'b1111110;

3'b001 : {a,b,c,d,e,f,g}=7'b0110000;

3'b010 : {a,b,c,d,e,f,g}=7'b1101101;

3'b011 : {a,b,c,d,e,f,g}=7'b1111001;

3'b100 : {a,b,c,d,e,f,g}=7'b0110011;

3'b101 : {a,b,c,d,e,f,g}=7'b1011011;

default : {a,b,c,d,e,f,g}=7'b1111110;

endcase

end

endmodule

module onoff(input clk, reset1, button_in, output wire out);

wire reset;

reg light_reg;

assign reset = ~reset1;

//synchronizer

reg button,btemp;

always @(posedge clk)

{button,btemp} <= {btemp,button_in};

//debounce push button

wire bpressed;

debounce db1(.clk(clk),.reset(reset),.noisy(button),.clean(bpressed));

reg old_bpressed;  //state last clk cycle

reg out0;

always @(posedge clk) begin

if (reset)

begin out0 <= 0; old_bpressed <=0;end

else if (old_bpressed==0 && bpressed==1)begin

//button changed from 0 to 1

out0 <= ~out0;

old_bpressed <= bpressed;

end

else if (old_bpressed==1 && bpressed==0)begin

//button changed from 1 to 0

out0 <= ~out0;

old_bpressed <= bpressed;

end

end

pwm pwm1(.clk(clk),.in(out0),.out(out));

endmodule

module debounce (

input clk,

input reset,

input noisy,

output reg clean

);

parameter COUNTER_WIDTH = 20;

parameter MAX_COUNT = 650000;

reg [COUNTER_WIDTH-1:0] count;

reg noisy_reg;

ys @(posedge clk or posedge reset) begin

if (reset) begin

count <= 0;

noisy_reg <= 0;

clean <= 0;

end else begin

noisy_reg <= noisy;

if (noisy_reg != noisy) begin

count <= 0;

end else if (count < MAX_COUNT) begin

count <= count + 1;

end else begin

clean <= noisy_reg;

end

end

end

endmodule

module pwm(

input wire in,

input wire clk,

output wire out

);

reg q;

always @(posedge clk) begin

q <= in;

end

assign out = in & ~q;

endmodule

2 电路级代码

module FSM(

input clk,reset,b0,b1,

output out,

output [3:0] display

);

parameter S_RESET = 3'b000;

parameter S_0 = 3'b001;

parameter S_01 = 3'b010;

parameter S_010 = 3'b011;

parameter S_0101 = 3'b100;

parameter S_01011 = 3'b101;

reg [2:0] state;

wire [2:0] next_state;

assign next_state[2] = (b0 == 0 && b1 == 0) ? state[2] : ((~state[2] & state[1] & state[0]) | (state[2] & ~state[1] & ~state[0])) & ~b0 & b1 & ~reset;

assign next_state[1] = (b0 == 0 && b1 == 0) ? state[1] : ((~state[2] & ~state[1] & state[0] & ~b0 & b1) |

(state[2] & ~state[1] & ~state[0] & b0 & ~b1) |

(~state[2] & state[1] & ~state[0] & b0 & ~b1)) & ~reset;

assign next_state[0] = (b0 == 0 && b1 == 0) ? state[0] : ((state[2] & ~state[1] & (~state[0] | b0 & ~b1)) |

(~state[2] & b0 & ~b1)) & ~reset;

always @(posedge clk or posedge reset) begin

if (reset) state <= S_RESET;

else state <= next_state;

end

assign out = state[2] & ~state[1] & state[0];

assign display = {1'b0,state};

endmodule